FLASHNET

Potential Builders PBX System

PREPARED BY: Ayaan Jamal

DATE: 21TH February 2026

1.1 Introduction

This deployment consists of a Yeastar S20 IP PBX connected directly to a ZLT X25 Max2 router. The Yealink W70B DECT base station is also directly connected to the same router.

The router provides:

- Internet connectivity (LTE via Airtel)

- DHCP services

- Local LAN communication between PBX and base station

1.2 Device Information

System Components:

- IP Phone: Yealink W71H

- Base Station: Yealink W70B

- PBX: Yeastar S20

- Internet Router: ZLT X25 Max2 (Airtel Home Wi-Fi)

Fig 1.2 Yealink S20

Fig 1.3 Yealink S20

Fig 1.2 Yealink W70B

Fig 1.1 Airtel Home Wi-Fi Router

Fig 1.4 Yealink W71H

2. IP Addressing Scheme

All the devices obtain connectivity directly from the router.

2.1 Recommended Configuration

| Device | IP Method | Recommended IP | Username | Password |
| --- | --- | --- | --- | --- |
| Router | Default | 192.168.1.1 | Admin | Admin |
| Yeastar S20 | Static | 192.168.1.69 | admin | Admin@2026 |
| W70B Base | Static | 192.168.1.70 | admin | Admin@123 |
| DHCP Pool | Automatic | 192.168.1.100-200 |  |  |

Fig 1.5 Topology of the System

2.2 Installing Active Directory Domain Services

Log into router (http://192.168.1.1).

Confirm internet and both LANS are active.

Enable DHCP.

5. Yeastar S20 Configuration

5.1 Access PBX

Connect to LAN.

Access via browser:

http://192.168.1.10

Change default admin password immediately.

5.2 Create SIP Extension

Navigate to:

PBX → Extensions → Add

Example:

Field	Value

Extension	101

Type	SIP

Password	Strong Password

Save and apply.

Repeat for additional users.

5.3 Access Web Interface

- Find IP via router DHCP list.

- Access:

http://192.168.1.69

- Login.

1.6 Login Page for W70B

1.7 Registering Wireless Handsets W71H

## Register Base to PBX

Go to: Account → Register

| Field | Value |
| --- | --- |
| Register Name | 500 |
| Username | 500 |
| Password | Extension Password |
| SIP Server | 192.168.1.10 |

Confirm registration.

Status must show: Registered

1.8 PBX Menu for Configuration of Extensions and to Connect to Base

| Customer Support Contacts |  |
| --- | --- |
| Email | cs@flashnet.co.tz, support@flashnet.co.tz |
| Call/WhatsApp | +255 22 2113687 (Call), +255-777- 988-883 (WhatsApp) |