FLASHNET

NetGate 1100 Documentation

PREPARED BY: Ayaan Jamal

DATE: 23TH February 2026

- Device Overview

The Netgate 1100 is a small-form-factor firewall/router appliance designed by Netgate.

Hardware Features

- 3x 1GbE switch ports (WAN/LAN configurable)

- ARM-based CPU

- Console access via micro-USB

- Pre-installed pfSense Plus

Default Settings

- LAN IP: 192.168.1.1

- Username: admin

- Password: pfsense

2. Device Information

Fig 1.1 NetGate 1100 Appliance

2.1 Physical Setup

- Step 1: Cabling

Connect WAN port to ISP modem.

Connect LAN port to PC or switch.

Power on device.

- Step 2: Access Web GUI

From a LAN-connected PC:

- http://192.168.1.1

Login with default credentials.

## 2.2 Physical Connectivity

To get started, connect the cables in this specific order:

- WAN Port: Connect to your Internet Service Provider (ISP) modem.

- LAN Port: Connect to your computer or a network switch.

- Console Port: (As seen in your screenshot) Connect to your PC via Micro-USB to use PuTTY.

- Power: Plug in the 12V adapter.

  - Note: Wait 4–5 minutes for the system to boot fully.

## 2.3 Initial Software Configuration

Once the boot loader menu (from your screenshot) finishes, you will see the pfSense Console Menu.

### 1. The Console Menu (Serial Access)

If you need to change settings before the web interface is accessible, use these common options:

- Option 2: Set Interface IP Address (Use this if you need to change the default 192.168.1.1).

- Option 3: Reset Admin Password (Useful if you're locked out).

- Option 4: Reset to Factory Defaults.

Fig 1.2 PF Sense serial controller through PuTTY

### 2. The WebGUI (The Main Interface)

Most users will manage the device here.

- Default URL: https://192.168.1.1

- Default Username: admin

- Default Password: pfsense

Fig 1.3 PFsense through the webGUI

## Section 3: Advanced Security Services (Packages)

The real power of the Netgate 1100 lies in its ability to run enterprise-grade security packages. In your demo, navigate to System > Package Manager.

- pfBlockerNG: This is the most popular add-on. It blocks ads, malicious domains, and entire countries (Geo-IP blocking) at the DNS level.

- Snort or Suricata: These are IDS/IPS (Intrusion Detection/Prevention Systems). They inspect packets for known "fingerprints" of hackers or malware.

- Surveillance: Explain that while the 1100 is an entry-level device, it can still handle light-duty traffic inspection for a home or small office.

## Section 4: Remote Access & Secure Connectivity

This is the "Work From Anywhere" section. Show how the Netgate 1100 creates a secure tunnel into the network.

### 1. WireGuard: The New Standard

Navigate to VPN > WireGuard.

- Speed: Emphasize that WireGuard is incredibly "light" on the CPU. For the Netgate 1100's ARM processor, this is the most efficient way to get high-speed VPN.

- Simplicity: Show how a QR code can be generated to instantly set up a smartphone to connect back to the office securely.

### 2. OpenVPN with Client Export

Navigate to VPN > OpenVPN > Client Export.

- This is a "Quality of Life" feature. Explain that pfSense makes a custom installer for Windows/Mac users.

Fig 1.4 OpenVPN Setup through wizard

### Tailscale (Package)

Many users love Tailscale for its "Zero Config" nature.

- Mention that Tailscale can be installed as a package on the Netgate 1100, allowing it to act as a Subnet Router or Exit Node. This allows you to route all your phone's cellular traffic through your home's encrypted pfSense connection with zero port forwarding.

## Section 5: Firewall Rules & Logic

The firewall reads rules from the top of the list to the bottom. Once a packet matches a rule, it stops looking.

### 1. The Anatomy of a Rule

Navigate to Firewall > Rules > LAN. Show a rule and explain the three main actions:

- Pass: The green checkmark. "You are allowed through."

- Block: The red "X". The packet is dropped silently. (Best for WAN to hide your presence from hackers).

- Reject: The orange "X". The firewall tells the sender "No, go away." (Best for internal LAN to stop apps from "spinning" while waiting for a timeout).

### 2. Best Practice: The "Default Deny"

Explain that by default, pfSense blocks everything coming from the internet (WAN) but allows everything leaving your network (LAN).

## Section 6: Traffic Shaping & Limiters

This is the "No More Lag" section. It's perfect for households where one person downloading a large game ruins the internet for everyone else.

### 1. The Traffic Shaper Wizard

Navigate to Firewall > Traffic Shaper > Wizards.

- Show the Multi-WAN / Dedicated Link wizard.

- It asks for your Upload/Download speeds and automatically prioritizes VoIP (Phone calls) and Gaming over "background" traffic like Windows Updates.

Fig 1.5 Pfsense Traffic Shaper

### 2. Bandwidth Limiters (The "Penalty Box")

Navigate to Firewall > Traffic Shaper > Limiters.

- Show how to create a "5Mbps Pipe."

- Demo Scenario: "If your roommate's BitTorrent is hogging the line, you can create a limiter and apply it to their specific IP. They will never be able to use more than 5Mbps, leaving the rest of the pipe open for your Zoom calls."

Section 6: Firewall Schedules

Navigate to Firewall > Schedules. This is a favorite for parents or office managers.

- How it works: You define a time range (e.g., "Bedtime" from 10:00 PM to 6:00 AM).

- Applying it: Go back to your Firewall Rules, edit a rule for the Kids_Devices alias, and select the "Bedtime" schedule.

- The Result: At 10:01 PM, the internet simply "turns off" for those specific devices, but stays on for yours.

Fig 1.5 Assigning the configuration file for default browser

| Customer Support Contacts |  |
| --- | --- |
| Email | cs@flashnet.co.tz, support@flashnet.co.tz |
| Call/WhatsApp | +255 22 2113687 (Call), +255-777- 988-883 (WhatsApp) |