FLASHNET

Huawei E-Kit Configuration and Testing

PREPARED BY: Ayaan Jamal

DATE: 16TH February 2026

This document provides the configuration steps for a Huawei eKit deployment using the Huawei eKit mobile phone application. The setup includes local LAN connectivity, PoE powering, access point onboarding, SSID creation, and Wi-Fi testing.

2. Device Information

| Device | Model | Function |
| --- | --- | --- |
| Switch | Huawei S380-L4P1T | LAN distribution + PoE |
| Access Point | Huawei AP266 | Indoor Wi-Fi |
| Access Point | Huawei AP326E |  |

Fig 1.3 Huawei AP266

Fig 1.1 Huawei S380-L4P1T

Fig 1.2 Huawei AP326E

2.1 Device Requirements

- Huawei eKit App installed on phone

- Internet connection on phone (for login)

- Router providing DHCP (or a DHCP server)

- Ethernet cables

- PoE switch ports available

Fig 1.4 Huawei eKit Deployment (before)

Fig 1.5 Huawei eKit Deloyment (after)

2.2 Connections

| Port | Connected To | Purpose |
| --- | --- | --- |
| GE0/0/1 | Router / Firewall | Uplink + Internet |
| GE0/0/2 | AP266 | Wi-Fi coverage |
| GE0/0/3 | AP326E | Wi-Fi coverage |

2.3 Adding the Devices (Switch + APs)

2.3.1 Adding the S380-L4P1T Switch

- In the Site dashboard select Add Device

- Choose Scan QR Code

- Scan the QR code on the switch

- Confirm device is added to the site

2.3.2 Adding AP266 and AP326E

- Select Add Device

- Scan QR code on:

  - AP266

  - AP326E

- Wait for the APs to appear online

Fig 1.6 Wi-Fi Management Page for Wi-Fi distribution

Topology design Example:

Fig 1.8 Topology example for Enterprises

| Customer Support Contacts |  |
| --- | --- |
| Email | cs@flashnet.co.tz, support@flashnet.co.tz |
| Call/WhatsApp | +255 22 2113687 (Call), +255-777- 988-883 (WhatsApp) |